Deflection Computation:
Moment Area Theorems (M.A.T.)

- From point D’, draw line D’t // OC’

- Consider deformed beam segment C’D’ on the neutral Axis (N.A.). The angle between 2 tangent lines at C’ and D’ is d τ 
Hence: C'OD' = OD't  = dτ

- Also curve line EFG is parallel to curve line C’D’ 
Hence: EFG ≡ ds

- FG = deformation = εds = (σcE) ds

Also, FG = Cd τ

- Hence σcEds=Cdτ

- MCIEds= C dτ

or dτ=MEI ds     =>  ∆τAB= MdsEI           => 1st M.A.T.

Notes:

The first and the second Moment Area Theorems (M.A.T.) will NOT directly give the rotation and displacement of a joint on the beam (or frame).

However, M.A.T will compute the change in angle (such as Δτ AB) between any 2 points (such as A’, B’) on the beam, and compute the vertical distance (such as d or Δ B/A) between 2 tangent lines (at A’, B’) on the beam.

Example 1: Use Moment Area Theorems (MAT), find the vertical displacement at point G (= δG?) and the rotations θa and θb at points A and B.

δG≡GG1 = (GG2) – G1G2 = (3/10 * ∆B/A) – ∆G/A

Where:

∆B/A =2nd MAT(Area under MEIdiagram)Between B and A * (S’ = distance from B to centroid of area)

Or: ∆B/A= (12*21Kft*7’)*(S’2=23*7’)+(12*21Kft*3’)*(S’1=7’+13* 3')

∆B/A= 595EI

And ∆G/A= (area under M/EI diagram)between B and A * (S’ = distance from to centroid of area)

Or ∆G/A=(12*21Kft*3’)*(S’=13*3’)=31.5EI

Thus: δG ≡ GG1= (310*595EI) - 31.5EI= 147EI = δG

θA = small angle =  ∆B/A10' = 59.5EI= θA

θB = small angle = ∆A/B10=(110){12*3’*21Kft*23 *3’+12 *7’*21Kft3'+13*7'}

Example 2:	Use M.A.T to find θA, θB,θC,  δC,  δmax

θA≅ ∆A/B18 ft ……………………. (1)

Where:

ΔB/A = (Area) * (S’= distance from B to centroid area)

ΔB/A =      (12 6’ * 120) * (S1’ = 12ft + 13  * 6ft)

+ (12 * 6’ * 30) * (S2’ = 6ft + 13   * 6ft)

+ (6’ * 120) * (S3’= 6ft + 12 * 6ft)

+ (12 * 6’ * 150) * (S4’= 23   * 6ft)

or ΔB/A= +14,040EI ----------------(1B)

So: θA= ∆B/A18ft= 780EI= θA

Similarly, θB≅ ∆A/B18ft  ……………….(2)

Where: ΔA/B= (Area under M EI diagram) Between A & B * (S’= distance from A to centroid area)

ΔA/B =   (12 * 6’ * 120) (S1’= 23 * 6’) + (12* 6’ * 30) (S2’= 6ft + 23 * 6ft)

+ (6’ * 120) (S3’= 6’ + 12* 6’) + (12* 6’ * 150) (S4’= 12ft + 13 * 6’)

ΔA/B= (360) (4) + (90) (10) + (720) (9) + (450) (14) = + 15,120EI

Hence:  θB= ∆A/B18= 840EI

Remarks:

- The area between A&B (of the M EI diagram) has been decomposed into three (3) triangulars and one (1) rectangular

- We can also calculate θB, as following:

θB= = θA+ ∆τAB   (= angle, rotation at A + change in angle between A&B

Where: ∆τAB≡(Area of M EI diagram) Between A&B = (12* 6’ *120) + (12*6’ * 30)

+ (6’ *120) + (12* 6’ * 150)

Or	∆τAB= 1620EI

Hence: θB= (θA=cw -780EI) + (+1620EI= ∆τAB) = +840EI= θB

ccw                                ccw

37

Also :	θC     =       θA     +    Δτ AC ______________________________ (3)

Where               θA        =   - cw 780EI

And 		 ∆τAB= 1st M.A.T   = (area under MEI diagram) between A&C

Δ τ AC = (12 * 6' * 120) = +  360EI

Because CCW (see picture previous page)

Hence     θC  = (-780EI) + (+360EI) = - cw 420EI= θc

- From the figure, one has:

δc ≡CC1=CC2- [C1C2]  _________________________________(4)

Or  δc =        = (6 ft18ft* ΔB/A) – [ΔC/A] _____________________________(5)

Where:  ΔC/A = 2nd M.A.T. = (area under MEI)Between C & A * (S'=distance from C to centroid area)

Or ΔC/A = (12* 6'*120) * (S'= 13* 6')  =  720EI

Hence:       δc = (618* 14,040EI)-720EI =  3960EI =  δc

- To find δmax (located at point M on the beam)

38

Notes 		Point M (with max. deflection) should be close to 30K load

Rotation at point M (θM) should be zero

We have:	θM=  θA +∆τAM ________________________________________(6)

0 = -720EI+ ∆τAM _______________________________________(7)

Where:

∆τAM=Area underMEIBetween A&M

∆τAM=12*6'*120+ (1/2 * x* [z=5x])+(120*x)

∆τAM=360+2.5x2+(120x) _____________________________(8)

Substitute Eq (8) into Eq (7) to obtain:

0=-780EI++2.5x2+120x+360EI→x=3.28 ft

Thus:		δmax≡MM2-M1M2=9.26ft18ft*∆B/A-[∆M/A]__________(9)

Similar triangles

39

Where: ΔM/A (Use 2nd M.A.T.) = (area underMEI)Between M & A *(S’= distance from M to centroid area)

ΔM/A = 12* 6'*120*(S1'=3.28'+ 13* 6')+ 12* 3.28*z=5*3.28'*(S2'= 13* 3.28')+ 120*3.28* (S3'= 12*3.28')

ΔM/A = (360) * (5.28') + (1.64) *(16.4)* (1.09) + (393.6) * (1.64) = 2575.6EI = 2575.6EI  (10)

Substituting Eq. (10) of Eq. (1B)  into Eq.  (9), to get:

δmax= 9.28ft18ft      *    14,040EI    -    [2575.6EI]

δmax= 4662.8EI

41

Example 3 Use Moment Area Theorems (M.A.T.), find the vertical deflection at joint A (δA) of the frame structure.

Moments Plotted on Tension Side

Strategies:

- Step1:  Compute ΔD/C (use 2nd M.A.T)

- Step 2:  θC ≅ ∆D/C9 ft

- Step 3:  Compute ∆τCB = Change in angle between tangent lines @ C & B (used 1st MAT)

- Step 4: θB = θC + ∆τC/B

- Step 5:  K ≅ (3ft) * θB

- Step 6:  Compute ΔA/B (use 2nd MAT)

- Step 7: δA=K+ ∆A/B

41

Step 1: 	          ∆D/C = (area of  MEI)D→C ) * (S’ = distance from D to centroid area)

= (12 * 90EI * 9ft) * (S’ = 23 * 9ft) = 2430EI

Step 2:                     θC  = 24309EI = 270EI

Step 3:                        ∆τCB = (area of  MEI)C→B  = 90EI * 3ft = 270EI

Step 4:                         θB  =(270EI)  + (+270EI) = (540EI)

Step 5:                          K = 3 * (540EI) = 1620EI

Step 6:          	  ∆ A/B = (area of MEI)A→B * (S’ = distance from A to centroid area)

= (12 * 90EI * 3ft) * (S’ = 23 * 3ft) = 270EI

Step 7:                          δA =  1620EI + 270EI = 1890EI=δA