Review of Statics

Equation of Condition(s)

(ΣMhingeB)BA=0=(ΣMhingeB)BC

Method 1: solving 4 simultaneous equations

ΣFx=0=+R1+R3≡f1R1,R3=0

ΣFy=0  =  ↑+R2+R4-20K-40K≡f 2R2,R4=0

ΣMA     =0=-20K15'-40K55'+R312'+R480'=0

f3(R3,R4)

(ΣMB)BA     =0=(20K)25'+R120'-R240'=f4(R1,R2)=0 ……(Eq.123)

Method 2:

ΣMA=0=f1(R3,R4)

Solve for R3,R4

(ΣMB)BC     =0=f2(R3,R4)

ΣMC=0=f3R1,R2=40K25'+20K65'-R112'-R2(80')

Solve for R1,R2

(ΣMB)BA=0=f4R1,R2=see Eq.(123)

Chapter 6 : Analysis of Structures

- Method of Joints

FBD of Joint B

*Each TRUSS member

is connected by 2

“hinge joints”

*A “hinge” join can

transfer forces, but NOT moments

*Truss members can only carry

Axial forces

- How to Compute Truss Member Forces

ΣFx= 0

Step 1:  Apply 3 equilibrium eqs.	    ΣFy= 0

To find 3 unknown reactions  ΣMz-axis=0

Step 2: At each joint which has 2 (or less) unknown member force(s),

apply 2 (force) equilibrium eqs         ΣFx=0

to find 2 (or less) member force(s)    Σfy=0

Step 3:  Repeat step 2 until all (or few desired) member force(s) are found.

- Method of Sections

This method is useful

if one has interests to

compute only few selected

(not ALL) member forces,

such as member forces

AB, DB, AD

Step 1: Apply 3 equilibrium eqs. for the entire structure to find 3 unknown reactions.

Step 2: Make an “intelligent and honest” cut section (1)-(1)

Then, decide “which side” of the cut to draw FBD, say LEFT side!

Step 3:  Apply 3 equilibrium eqs, using LEFT – FBD:

∑MD=0⇒f1FAB,Ex,Ey=0→solve for FAB

∑Fx=→+0⇒f2Ex,FDC,FDBcosα,FAB=0→solve for  FDC

↑

∑Fy=↑+0⇒f3Ey,-18k,FDBsinα=0→  solve for  FDB

Notes:  The remaining member force FADcan be found by:

*Apply Method of Joint at joint A:	 ΣFx=0

Solve for FAD,FAE

Σfy=0

*or make another cut section (2)-(2), use LEFT-FBD, and

ΣM@A=0=>f4FAB,FAD,FED,Ex,Ey=0=>Solve for FED

ΣFx=0=>f5Ex,FED,FADcosβ,FAB=0

Solve for FAD

ΣFy=0=>f6Ey,FADsinβ=0

C1) Quick Ways to determine zero forces in truss structure

At Joint B:

ΣFy=↑+ 0  = >   0 = FBD

At Joint D:

ΣFy= 0= FDAcosα = > FDA = 0

Rules

‘‘If we have 3 TRUSS members joining at a common joint (say, joint D), and if there are NO FORCES applied at the common Joint D, if two members form a 180o angle, such as members DB and DC, then the remaining member (such as member DA) should have Zero-force !’’

At Joint B;

ΣFy=↑+ 0=-FBA

At Joint B:

ΣFx=→+ 0 = FBA

C2) Multi-Force (Beam, Frame) Members

Notes:

- Members AD and CF are multi-force members

- Member BE is a 2 force (axial) member

Problem 6.4

Using the method of Joints, determine the force in each member of the truss shown. State whether each member is in tension or compression

Problem 6.8

Using the method of joints, determine the force in each member of the truss shown.   State whether each member is in tension or compression.

SAMPLE PROBLEM 6.2

Determine the force in members EF and GI of the truss shown.

SOLUTION:

Free-Body: Entire Truss. A free-body diagram of the entire truss is drawn; external forces acing on this free body consist of the applied loads and the reactions at B and J. We write the following equilibrium equations.

MB=0:

-28 kips8 ft-28 kips24 ft- 16 kips10 ft+J32 ft=0

J = +33 kips               J=33 kips ↑

→+Fx=0:                    Bx + 16 kips = 0

Bx = -16 kips 	     Bx= 16 kips ←

MJ=0:

28 kips24 ft+28 kips8 ft- 16 kips10 ft-By(32 ft)=0

By= +23 kips  By=23 kips ↑

Force in Member EF. Section nn is passed through the truss so that it intersects member EF and only two additional members. After the intersected members have been removed, the left-hand portion of the truss is chosen as a free body. Three unknowns are involved: to eliminate the two horizontal forces, we write

+↑Fy=0:          +23 kips-28 kips-FEF=0

FEF= -5 kips

The sense of FEF was chosen assuming member EF to be in tension; the negative sign obtained indicates that the member is in compression. FEF=5 kips C

Force in Member GI. Section mm is passed through the truss so that it intersects GI and only two additional members. After the intersected members have been removed, we choose the right-hand portion of the truss as a free body. Three unknown forces are again involved; to eliminate the forces passing trough point H, we write    
MH=0:               (33 kips)8 ft- 16 kips10 ft+FGI10 ft=0

FGI= -10.4 kips         FGI=10.4 kips      C

Chapter 7:

Forces in Beams and Cables

- Sign Conventions :

M= +

“Positive’’ end-moments (because the “rice” still

Remains inside the “bowl”)

M=   -

“Negative” end-moments (because the “rice” will fall-off

From the “bowl”, hence no rice left to eat! )

FBD of beam segment AC, where V1& M 1 are internal shear force & bending moment in cut

section (1) – (1)

- Relations Between (Distributed) Loads “w”, Interval Shear Force “v”, and Bending Moment

Consider the FBD of beam segment between cut-sections (2) – (2) and (3) –(3).

For Equilibrium, one has:

∑Fy=↓+0=-V+w∙∆x+V+∆V

0 = w∆x∆x+∆V∆x ∆V∆x=-w

Let ∆x→0 hence: dV dx =-w

∑M(2)=0+=-M-w∆x∆x2+M+∆M-(V+∆V)∆x

0=-w∆x2+∆M∆x-(V+∆V)

Let ∆x→0; hence ∆V→0; and above eq. becomes:

0=+dMdx-V; Or   dMdx=V

Or (2)(3)dM=(2)(3)dx

M(3)-M(2)=Area under shear curve, between cut-sections (2) and (3).

Basic Examples About Shear & Moment Diagrams

Example 1 (Concentrated Force Applied)

Quick Ways To Draw   V   &   M   Diagrams

To draw the internal shear diagrams:

“Look at the given beam structure, observe how the shear forces applied on the beam. Then, draw the   V   diagram accordingly”

To draw the internal moment diagrams:

Step 1: Refer to the shear diagram, and compute the areas under the shear diagram sections

Step 2: Compute the initial value of internal moment (including the proper sign) say M (1)

Step 3: Apply the formula: M (2) –M (1) =area under V diagram,

Between sections (1) & (2) to compute M (2)

Step 4: Decide how to connect the (two) dots in the M diagram

by referring to Eq. dMdx=V; or M (2) –M (1) =(1)(2)Vdx

Problem 7.34

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution: (a)

FBD Beam:

- MB=0 :

(.6 ft)(4 kips) + (5.1 ft)(8 kips) + (7.8 ft)(10 kips) – (9.6 ft) Ay= 0

Ay= 12.625 kips ↑

↑ΣFy= 0 :12.625 kips-10 kips-8 kips-4 kips+B=0

B = 9.375 kips  ↑

Along AC:

↑ΣFy= 0 : 12.625 kips-V=0

V= 12.625 kips

ΣMj=0 :M-x12.625 kips=0

M = (12.625 kips) x

M = 22.725 kip.ft at C

Along CD:

↑ΣFy= 0 :12.625 kips-10 kips-V=0

V = 2.625 kips

- ΣMk=0 :M+x-1.8 ft10 kips-x12.625 kips=0

M = 18 kip.ft + (2.625 kips)x

M = 29.8125 kip.ft at D (x=4.5 ft

Problem 7.34 Continued

Along DE:

↑ΣFy= 0 : (12.625 – 10 – 8) kips – V = 0, hence V = -5.375 kips	
ΣML=0 :M+x18 kips+ 2.7 ft+x110 kips- 4.5 ft+ x112.625 kips =0

M = 29.8125 kip.ft - (5.375 kips) x1

M = 5.625 kip.ft at E (x1=4.5 ft)

Along EB:

↑ΣFy= 0 :  V+9.375 kips=0                                        V=9.375 kips

- ΣMN= 0 :  x29.375 kip- M=0

M = (9.375 kips) x2

M = 5.625 kip.ft at E

(b) From diagrams:

Vmax = 12.63 kips on AC

Mmax =  29.8 kip.ft at D

Problem 7.35

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

(a)

FBD Beam:		ΣME= 0:

(1.1 m)(540 N) – (0.9 m)Cy+0.4 m1350 N- 0.3 m 540 N=0

Cy=1080 N ↑

↑ΣFy= 0:                Ey=  1350 N ↑

Along AC:

↑ΣFy= 0:  -540 N- V= 0

V = -540 N

ΣMj= 0:  x540 N+M=  0

M = -(540 N)x

Along CD:

↑ΣFy= 0:  -540 N + 1080 N - V= 0         V = 540 N

ΣMK= 0:  M+0.2 m+ x1540 N- x1(1080 N)=0

M = - 108 N.m + (540 N) x1

M = 162 N.m at D (x1=0.5 m)

Problem 7.35 continued

Along DE:

↑ΣFy= 0:    V+ 1350 N – 540 N = 0   V = -810 N

ΣMN= 0:  M+x3+ 0.3 m540 N- x3(1350 N)=0

M=-162 N.m + (810 N)x3

M=162 N.m at D (x3=0.4)

Along EB:

↑ΣFy= 0:    V – 540 N = 0                                       V = 540 N

- ΣML= 0:  M+x2540 N=0                    M = -540 Nx2

M=- 162 N.m at E ( x2=0.3 m)

(b) From diagrams:

Vmax = 810 N on DE

Mmax= 162.0 N.m at D and E

Problem 7.36

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

- FBD Beam

a = 1 m       ΣFx= 0                   Bx=0

ΣFy= 0       -1.5 kN+2kN – 4 kN+5kN-By=0

By= 1.5 kN

ΣMB= 0 :

a[4(1.5 kN)– 3(2 kN) + 2(4kN)– 1(5kN] – MB= 0

MB= (3kN)a = 3kNm

Along AC:

ΣFy= 0  -1.5kN – V= 0

V= -1.5kN

ΣMJ= 0: M-x(1.5 kN) = 0                 M(1 m )= - (1.5)x

Problem 7.36 Continued:

Along CD:

↑ΣFy= 0   : -1.5 kN + 2kN – V = 0             V= 0.5 kN

Mk = 0   : M+ x(1.5 kN) – (x-1m)(2kN) = 0

M= -2 kNm + (0.5 kN)x       M(2m) = -1kNm

Along EB:

↑ΣFy= 0       V-1.5 kN=0        V= 1.5kN

ΣML= 0 :       -M-x1(1.5kN) – 3kNm = 0

M = -3 kNm –(1.5 kN) x1

M(1m) = -4.5 kNm

Along DE:

↑ΣFy= 0  :  V + 5 kN – 1.5 kN = 0       V= -3.5 kN

Also M is linear here

- Vmax=3.50 kN along DE

Mmax= 4.50 kNm at E

Problem 7.37

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

- FBD Beam

ΣMA= 0:

-1.3 m(1.8kNm(2.6 m)]-(1.6 m)(4 kN)+(4 m) B=0

B = 3.121 kN↑

↑ΣFy= 0:

Ay-1.8 kN?m2.6 m- 4 kN+3.121 kN=0

Ay=5.559 kN ↑

Along AC:

↑ΣFy= 0:   5.559 kN-1.8 kN/mx-V=0

V=5.559 kN-1.8 kN/mx

ΣMJ= 0:   M+X2[(1.8 km)x]-x(5.559 kN)=0

M=(5.559 kN)x –(0.9 kN/m) x2

Problem 7.37 Continued

Along CD:

↑ΣFy= 0:   5.559 kN-x1.8 kN/m-4 kN-V=0

V=(1.559 kN)-1.8 kN/mx

- ΣMK= 0:   M+x-1.6 m4 kN+x2[(1.8 km)x]-x(5.559 kN)=0

M=6.4 kN.m+1.559 kNx-(0.9kNm)x2

Along DB:

↑ΣFy= 0:              V+3.121 kN=0

V = -3.121 kN

- ΣML= 0:            -M+x1(3.121 kN)=0

M = (3.121 kN) x1

Vmax = 5.56 kN at A

Mmax= 6.59 kN.m at C

Problem 7.38

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

- FBD Beam:

←∑Fx=0:             Ax=0

↑ΣFy= 0   Ay+(2m)(24kN/m)-48kN-8kN=0

Ay=8kN↑

- ∑MA=0 :  MA+(1m)(2m)(24kN/m)-(3.5m) (48kN)-(2m)(8kN)=0

MA=152kN∙m

Along AC:

↑ΣFy= 0:  8kN+x24 kN.m-V=0

V=8 kN +(24 kN/m)x

- ΣMJ= 0:  M+152 kN.m

-x8 kN- x224kNmx=0

M = (12 kN/m) x2+(8 kN)x - 152 kN.m

Problem 7.38 Continued

Along DB:

↑ΣFy= 0:           V-8kN=0

V = 8kN

ΣMK= 0:          M+ x1(8 kN)=0               M = - (8kN) x1

Along CD:

↑ΣFy= 0:           V-48 kN-8 kN=0,             V=56 kN

- ΣML= 0:           M+x1- 0.5 m48 kN+x18 kN=0                 M=24 kN.m-(56 kN)x1

Vmax=56.0 kN along CD

Mmax=152.0 kN.m at A

Problem 7.39

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

(a) By symmetry, Cx = 0, and

Cy=Gy= 12[212lbin.10 in+ 2100 lb+150 lb]

Cy=Gy=295 lb↑

Along AC:

↑ΣFy= 0: -12lbin.x-V=0

V= -12lbin.x

ΣMJ= 0:   M+ x212lbin.x=0         M= -(6lbin.)x2

Along CD:

↑ΣFy= 0: -12lbin.10 in.+295 lb-V=0

V= 175 lb

- ΣMK= 0:   M+x-5 in.12lbin.10 in.-x-10 in.295 lb=0

M= -2350 lb.in. + 175 lbx

Problem 7.39 Continued

Along DE:

↑ΣFy= 0: -12lbin.10 in.+ 295 lb-100 lb-V=0,           V=75 lb

ΣMN= 0:  M+x-16 in.100 lb- x-10 in.295 lb+ x-5 in.12lbin.10 in.=0

M= -750 lb.in+75 lbx

- Complete diagrams using symmetry.

Vmax=175.0 lb along CD and FG

Mmax=900 lb.in at E

Problem 7.71

For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the maximum absolute values of the shear and bending moment.

Solution:

- ΣMA= 0: -3m9 kN- 27 kN.m-9m12 kN+ 12mF+16.5 m  3 kN+ 22.5 kN.m=0

F=7.5 kN ↑

ΣFy= 0:     Ay- 9 kN-12 kN+7.5 kN+3 kN=0

Ay =10.5 kN ↑

Shear Diag:

V is piecewise constant, with jumps at A, C, E, F, and B, equal to the forces there.

Moment Diag:

M is piecewise linear with jumps at D and B equal to the couples there.

MC=10.5 kN3 m=31.5 kN.m

MD-=31.5 kN.m+1.5 kN3 m=36.0 kN.m

MD+=36 kN.m+27 kN.m=63 kN.m

ME=63 kN.m+1.5 kN3 m=67.5 kN.m

MF=67.5 kN.m-10.5 kN(3m)=36 kN.m

MB-=36.0 kN.m-3 kN4.5 m=22.5 kN.m

- From the diagrams:

Vmax=10.50 kN along AC and EF

Mmax=67.5 kN.m at E

Problem 7.74

For the beam shown, draw the shear and bending-moment diagrams, and determine the maximum absolute value of the bending moment knowing that (a) P = 7 kips, (b) P = 10 kips.

Solution:

↑ΣFy= 0:  Ay(2kipsft)(8 ft)- 3kips + 7kips = 0

Ay=12 kips ↑

ΣMA= 0: MA+ 4 ft2kipsft8 ft

- 14 ft3 kips- 20 ft7 kips=0

MA= 34 kip.ft

Shear Diag:

V jumps to 12 kips at A, then decreases at 2 kips/ft to -4 kips at C to D.

V drops 3 kips to -7 kips from D and B and jumps 7 kips to zero.

Note:

V = 0 where 12 kips – (2 kips/ft) x = 0, x = 6 ft.

Moment Diag:

M jumps to 34 kips.ft at A and then increases with decreasing slope to

34 kip∙ft + (12 kips) (6 ft) = 70 kip∙ft at E, and decreases by (4 kips) (2 ft) = 4 kip∙ft, to 66 kip∙ft at C. M then decrease by (4 kips)(6 ft) to 42 kip∙ft at D, and by (7 kips) (6 ft) to zero at B.    
Mmax=70 kip.ft  at E

↑ΣFy 0:  Ay- 2kipsft8ft- 3 kips+10 kips=0

Ay=9 kips ↑

ΣMA= 0: MA+ 4 ft2kipsft8 ft+14 ft3 kips

- 20 ft10 kips=0	    	
MA= 94 kip.ft

Shear Diag:

V jumps to 9 kips at A, then decreases, at 2 kips/ft, to -7 kips at C to D, drops 3 kips to -10 kips from D to B and jumps 10 kips to 0.

Note: V = 0 where 9 kips – (2 kips/ft) x = 0, x = 4.5 ft.

|  | Problem 7.76 For the beam and loading shown, (a) draw the shear and bending-moment diagrams, (b) determine the location and magnitude of the maximum absolute value of the bending moment. |
| --- | --- |

Solution:

+ΣMA=0:

6mBy- 1 m20kNm2 m- 24 kN.m-5m20kNm2m=0

By=44 kN ↑

↑ΣFy=0:  Ay- 220kMm2 m+ 44 kN=0

Ay=36 kN ↑

Shear Diag:

V jumps to 36 kN at A, then decreases with slope -20 kN/m to -4 kN

at C, is constant to E, then decreases with slope -20 kN/m to -44 kN

at B.

Note: V = 0 at F where 36 kN – (20 kN/m)x = 0, x = 1.8 m.

Moment Diag:

Starting at zero M increases with decreasing slope to (36 kN)(1.8 m) = 32.4 kN∙m at F, decreases by (4 kN)(0.2 m) to 32 kN∙m at C, then with slope -4 kN to 28 kN∙m at D, where it jumps to 52 kN∙m, M decreases with slope -4 kN to 48 kN∙m at E, then with increasingly negative slope by (2 m) to zero at B.

Problem 7.79

Solve Prob. 7.78 assuming that the 2-kip force applied at E is directed upward.

Problem 7.78: For Beam AB, (a) draw the shear and bending-moment diagrams, (b) determine the location and magnitude of the maximum absolute value of the bending moment.

- Note: The 2 kip force at E has been replaced by the equivalent force and couple at C.

- MAB=0:  -6 ft1kipft12 ft+ 12 ft2 kips- 8 kip.ft+24 ftDy- 32 ft1 kip=0

Dy= 113 kips ↑

↑Fy= 0:  Ay- 1kipsft12 ft+ 2 kips- 113 kips-1 kip=0

Ay= 223 kips ↑

Shear Diag:

Starting at 223 kips at A, V decreases with slopes – 1 kip/ft to -143 kips at C, jumps 2 kips and remains constant at -83 kips to D, jumps 113 kips and remains constant at  1 kip to B, drops to zero.

V = 0 at, where 223 kip – (1 kip/ft)x = 0, x = 223 ft

Moment Diag:

Starting from zero, M increases with decreasing slope to 12223 kips(223  ft) = 26.889 kip.ft at F.

M increases by  12143 kips(143  ft) to 16 kip.ft at C, jumps to 24 kip.ft, decreases with slope -83  kips to – 8 kip.ft at D, and finally increases with slope 1 kip to zero at B.

Mmax=26.9 kip.ft at F (7.33 ft from A)