DEFLECTION BY VIRTUAL WORK METHOD

- Principle of virtual work for Rigid Bodies.

Consider a truly rigid body which is in static equilibrium under a system of forces Qi.

Suppose this rigid body is TRANSLATED a small amount (say δ0) by some other cause (which is independent of Qi force system).

Since δ0 is small all Qi forces are assumed to maintain the same position and direction relative to the rigid body and to each other, and hence to maintain EQUILIBRIUM during this rigid translation:

QnX  += 0             (10)

QnY  ↑+= 0             (11)

M0  = 0 =  (Qnx*yn)+(-Qny*xn)  (12)

Work done by the

Qi forces = WQ = (QnxδOx+ QnyδOy)=(δOx)Qnx+ (δOy )Qny        (13)

Zero

WQ=0                 (14)

43

Similarly , if  the  rigid  body  is  ROTATED  a  small amount  ∝0 by  some  other  cause  (which  is independent of  Qi   forces),  then:

Since: α0 is a small angle ⇒ OBB'≈90o

Hence OBH≡γ≅ HB'B

We have:

OB=xn2+yn2   And  BB'≈OBαorad=xn2+yn2 (αo)

HB'=BB'*cos(γ)=BB'*ynOB=xn2+yn2 αo*ynxn2+yn2=∝oyn

BH=BB'*sin(∝)= BB'*xnOB=xn2+yn2  αo*xmxn2+yn2=∝oxn

Now,

WQ=(Qnx*HB'-Qny*BH)=(Qnx*∝oym-Qny*∝oxn)……(15)

WQ=∝o(Qnxyn-Qnyxn)=0

Zero (see Eq. 12)

\

44

Principal of Virtual Work for Deformable Bodies:

Step 1:			This deformable body is in

Equilibrium under the applied

virtual forces Qi and support

reactions.

Step 2:	Now ,  suppose  the  body  is  subjected  to  a  small  change

in  shape,  caused  by  another  load  P  system  (which  is  independent

from  Qi  system)

Step 3:                         Any INTERIOR particle, with dimensions dx and dy, will be displaced in its boundary and will have its virtual work to be canceled

by its adjacent particle(s).

Step 4:

Virtual work done on the EXTERIOR (or boundary)

Particle is given by:  dwS=dwR+dwd…………….(16)

Where:  	dwR=due to RIGID body Translation & Rotation = 0 (see Eq. 14 & Eq. 15)

dwd=due to deformable body

Hence:                       dwS= dwd⇒∫dwS=∫ dwd     or   ws=wd………(17)

45

(C) How to Evaluate WS & Wd in Eq. (17)  ?

In  Eq.  (17),      one       has:	    Ws  =   Wd

Where WS & Wd represents the External Virtual Work (VW),

and the Internal V.W., respectively

Ws =  Q*δ = (“Virtual” Force or Moment) *(“Real” Displacement, or Rotation) ……..(18)

The internal V.W. (=Wd) of deformation can be computed as:

Wd=(Wd)Axial+(Wd)Bending+(Wd)Torsion+(Wd)Shear ………..(19)

With:

(Wd)Axial=FQ*∆Lreal=FpLAE+αT*∆T*L+∆LFabrication Error…(19a)

Length

Strain

“Real” Displacement

Stress

Area

“Virtual” Force

(Wd)Bending=All memberss=0Ly=-h2h2MQyI(bdy)*MpyIE*ds=OLMQ*MpdsEI..(19b)

Since:

y

z

b

h2

-h2

y=-h2h2 y2bdy=by33-h 2h2≡I

(Wd) Torsion = (TQ = Virtual Torque)* (TPLJG  = Real angle of twist).  .  .  .  .  .   (19c)

(Wd) shear =k s=0LVq   (VpAG )  ds        .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . .  .    (19d)

Where k=a constant, depending on the cross-sectional shape of the beam

Important Notes

- The nature of the applied “VIRTUAL”  Loador Moment Force  does depend on

the “real” displacement or Rotation Translation  that  we want to compute

(as will be demonstrated through many V.W. examples).

- In Eqs. (19a - 19d ), the subscripts Q and P represents “Virtual” and “Real”

loads, respectively.

Example 1:  2-D Truss with Applied Load

Units used = Kips, Inches

E   = Young Modulus (K/in2)

For the truss (shown in Fig. a), find the Horizontal & Vertical displacements at Joint C.

Solution for Example 1

- For finding the Horizontal displ. at Joint C (∆choriz); one applies Virtual Load Q1    (Fig. b)

- For finding the Vertical displ. at Joint C(∆cVert.); one applies Virtual Load Q2          (Fig. c)

- From V.W Formulas (see Eqs. 17;  19 a →19d ) , one has ;

Ws + Wsettlement = Wd 	       (Q1 = 1 K→)(∆choriz ?? )+ Wsettlement = All membersFQi (Fp LAE)i .. (20)

(Q2 = 1k ↓    ) (∆cVert.?? ) + Wsettle  = iFQ2i (Fp LAE)i ……………………   (21)

- From Static equilibrium , and applying Methods of Joints in figures (a + b + c ), one gets:

| Member | Length ≡ L | Area ≡ A | LA | Fp | FQ1 | FQ2 | FQ1FpLA | FQ2FpLA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB | 150 | 2.5 | 60 | 50 | 5/3 | 5/4 | 5,000 | 3,750 |
| BC | 90 | 1.5 | 60 | 30 | 1 | 3/4 | 1/800 | 1,350 |
| CD | 150 | 5.0 | 30 | -50 | 0 | -5/4 | 0 | 1,875 |
| BD | 120 | 4.0 | 30 | -40 | -4/3 | -1 | 1,600 | 1,200 |
| AD | 90 | 3.0 | 30 | -30 | 0 | -3/4 | 0 | 675 |
|  |  |  |  |  |  |  | 1=8,400 | 2=8,850 |

- Hence: ∆choriz. = +8400E and   ∆cVert. = 8850E

Example 2:   2-D Truss with Applied Load

Find the “relative displacement” of joints C and B.

Area of each bar = 10 in2 = A

Young Modulus = 30,000 kin2 = E

WS + Wsettlement=Q=1K∆CB+ 0=iFQiFpLAEi

= {(-35*75K*180'')+(1K*-125K*300'')+(-4K5*100K*240'')}*(1AE)

= -5400AE

Thus:  ∆CB=-5400AE   (or   ∆CB=5400AE   )

Example 3:  Plane Truss

(with Settlements, Fabrication Errors)

The truss shown is subjected to support settlements

At joints A {2''↑3''→  and B  {3'' ↓2''→

What should be the length of member AD in order to have  (∆D)Horizontal = 0’’?

Given cross-sectional area of each bar = 20 in2 = A

Young Modulus = 30,000 Kin2 = E

Solution

- Due to settlements’ effects, horizontal displ. at joint D can be computed as:

Ws + Wsettlement= Wd    zero

Q=1k →  ∆DHoriz.1++Ax=+1k(+3'')+(-Ay=-4k3)(2")+(By=4k3)(-3")=0

Thus: ∆DHoriz.1 =+(113)in , or joint D moves to the right.

- In order to prevent the Horiz. displ. at joint D (due to settlement), length of member AD

has to be adjusted, so that joint D will move back to the left, by the same amount.

(Ws=1K*-11in3)+Wsettle=FQAD=5K3∆LFabricationError for AD??→(∆LFab.Err)AD =-11"5

0

Example 4:  Plane Truss with Temperature, Pre-strain (or Fabrication Error), Support settlements … effects

Given: E=30,000 kin2

α =6.5*10-6inin°F

Fy =0.5k

K

F

Find: Vertical displ. at joint E.

- Due to temperature of bars AB, BC, DE & EF is raised by 100°F

Ws + Wsettlement= Wd

Q=1k ↑∆Evert.+0=iFQi (α∆T*L)i where i=bars AB, BC, DE, EFFQAB=0.5k=FQBCFQDE=0k=FQEF

So: ∆Evert.=+6.5*10-3ft (or moves upward)

- Due to Pre-strain (or Fabrication Error)

Bars AB, BC, DE, EF, AD, BE, CF are 2" too short.

Bars AE, CE are  2=1.414” too short

WS+Wsettle=Wd

Q=1K↑∆Evert+0=ΣFQi∆LPi where i=AB, …, CF; AE, CE

=0.5k-2'')+…+0.5k(-2''+-1k2-2"+-1k2-2"

AB		     CF                     AE                        CE

So: 	∆Evert=-2" (or moves downward)

- Due to settlements at Joint D2"←1" ↓ , and at Joint F 3"→4" ↓

WS          +     Wsettle              =        Wd

Q=1K↑(∆E)vert+ Dy=-0.5k-1'')+(Fy=-0.5k)(-4''=  0

So: 	∆Evert=-2.5" (or moves downward)

- Due to all, combined effects

(∆E)vert= +6.5*10-3ft*12"ft+ -2'')+(-2.5''=-4.42"

Example 5: V.W on Beam

Find the vertical deflection ∆c at mid-point of the beam?

- Use left-side of section (1)-(1) as free Body Diagram (FBD)

ΣM1-(1)=0=Mp1-Ay=wL2x1+wx1(x12)

Hence: Mp1=wLx12- wx122   for 0 ≤ x1 ≤L2

Similarly:

Mp2=wLx22- wx222

for L2 ≤ x2 ≤L

- Use left-side of section (1)-(1) as FBD:

ΣM1-1=0=MQ1-Ay=0.5Kx1

So: MQ1=0.5x1 for 0 ≤ x1 ≤L2

- Use left-side of section (2)-(2) as FBD:

ΣM2-2=0=MQ2-Ay=0.5Kx2+1kx2-L2

So: MQ2=-0.5x2+L2   For  L2 ≤ x2 ≤L

Now, applying V.W. formula (see Eq. 19 b)

WS+Wsettle=MQ*MpEI dx

Q=1k↓∆c??+0=x1=0L2MQ1*MP1EI dx1+x2=L2LMQ2*MP2EI dx2

Or: 	∆c=x1=0L2(0.5x1)*wLx12- wx122EIdx1+ x2=L2L-0.5x2+L2*wLx22- wx222EIdx2

Hence: ∆c=+5wL4384EI (or point C will move downward, as we have assumed for direction of Q)

`

Example 6:  More on V.W. for Beam

For the same beam (Shown in Example 5), find the rotation (θB) at Joint B

∑M 1-(1 ) = 0 =  MQ1 – (Ay= 1L)kip(x1)

Hence:    MQ1 = x1L  for   0 ≤ x1 ≤ L

Applying V.W. formula (See Eq. 19b):

WS     +     Wsettle.      =      Wd

(Q = 1k-in)θB  + 0 =   MQ MpEI dx

Or          θB  =  x1=0L(x1L)*(wLx12- wx122)EI dx1

Note:

The Mp1 computed earlier ( see Example 5 ) can also be used for entire beam  ( or  0 ≤ x1 ≤ L  )

Example 7: V.W. on Frame

Given: E =  30,000k/in2 ;   I = 288 in4

Find: vertical deflection at point C? Rotation at point C ?

- Real Load, P-System

- Free body Diagrams (for P-system)

∑M 2–(2)= 0 = Mp2+ (1000k) (x2) - 600kft

So:

Mp2= 6000kft-1000x2

0’ ≤  x2 ≤ 10’

∑M 1-( 1) = 0 = Mp1 - (600k)(x1)

So:

Mp1 = 600x1

0’ ≤  x1 ≤ 10’

Note: To find vertical deflection at point C, we could have also applied a “Virtual” vertical load Q1= 1000k at point C.

Hence, Fig. (b) can also be used for Q1-system !

To find vertical deflection (∆c) at joint C

Ws    +   Wsettle   =   Wd

(Q1=1000K)(∆ftc)   +   0Kft               =MQ* MpdxEI

Or    (1000) ∆cft  =   x1=010MQ1* Mp1EIdx1 +    x2=010MQ2* Mp2EIdx2

=   010(Mp1)EI2dx1+  010(Mp2)2EIdx2

(1000)∆cft  =   010(600x1)2EIdx1  +  010(6000kft-1000x2)2EIdx2

Thus:         ∆cft  = (11000)  {010(600x1)2EIdx1  +  010(6000-1000x2)2EIdx2  }

M2-(2)= 0=MQ2+ 1K-ft

So:

MQ2= -1Kft

Valid for 0’ ≤  x2 ≤ 10’

M1-(1)= 0=MQ1+ 1Kft

So:

MQ1= -1Kft

Valid for 0’ ≤  x1 ≤ 10’

(C)- Virtual Load (Q2-System) and FBD of members AB, BC.

To find ROTATION  at joint C

Ws    +   Wsettle   =   Wd

Q2=1KftθC+ 0Kft       =  MQ*MpdxEI

Or: θC= x1=0'10'MQ1*MP1dx1EI+x2=0'10'MQ2* MP2dx2EI

Or: θC= 010(-1Kft)*(600x1)EIdx1+010(-1Kft)* (6000Kft-1000x2)EIdx2

Example 8: V.W. for a Determinate Beam

Find the vertical deflection of joint B.

Ws=Wd

1kδBV+ Wsettle=0= MQ1MP1dxEI (8.1)

Use left side of cut section 1)-1) as FBD, then:

M1= 0=MP1-24kft ⟹MP1=24kft   (8.2)

Use left side of the cut section 1)-1) as FBD, then:

M1= 0=MQ1+ 1kx- 12kft

Hence:	MQ1=12-x         (8.3)

Substitute Eqs. (8.2, 8.3) into Eq. (8.1), one gets

δBV= 12-x24(dx)EI = …

(b)- Virtual Load (Q-System)

Example 9:   V.W. for Determinate Beam

Find the vertical deflection at joint B? (Refer to example 8)

Ws=Wd

Q=1k↑δBV+Wsettle=0(Mq1(MP1=Mq1)EIdx

δBV= Mq12dxEI=012    12-x2EIdx=…

Example 10: V.W. for simply support, Determinate Beam

Find the ROTATION at joint A.

Use left side of cut section 1)-1) as FBD;

M(1)=0=MP1-2Kx⟹MP1=2x..(10.1)

M(1)=0= MQ1+1kft-(112)(x)

Hence: MQ1= x12-1   ……….…(10.2)

Use V.W:

Ws+ Wsettle=Wd

(Q=1Kft⤣)θA+ 0= MQ1MP1EIdx

θA=012(x12-1)2xdxEI=…

Example 11: V.W. for Simply Support, determine beam.

Find the rotation at Joint A.

Use left side of cut section (1)-(1) as FBD;

M1=0= MP1+1kft-112x

Hence: MP1= x12-1≡ MQ1

Apply VW:

Ws+ (Wsettle=0) = Wd

(Q=1kft⤣)θA+ 0= MQ1MP1dxEI

θA= 012(x12-1)2dxEI=…

Example 12 

Find the vertical displacement at joint B?

Use left-side of cut section (1)-(1) as FBD.

Mcut( 1)=0= MP1-Ay(x)

So: MP1= Ayx=-0.5x ≡ MQ1

Real (P-System) ≡ Virtual (Q- System)

Apply V.W.

Ws+ Wsettle = Wd

(Q=1K↑)δBV+ 0= MQ1MP1dxEI

δBV= 0L(-0.5 x)2dxEI= …